Agricultural Unpaved Road Dust Emissions

(Revised March 2004)

EMISSION INVENTORY SOURCE CATEGORY Miscellaneous Processes / Farming Operations

EMISSION INVENTORY CODES (CES CODES) AND DESCRIPTION _________________________________ (Agricultural Unpaved Road Dust Emissions)

# METHODS AND SOURCES

The land preparation source category includes estimates of the airborne soil particulate emissions produced by the unpaved roads in the agricultural lands. Dust emissions are produced by the mechanical disturbance of the soil by the  vehicles, people, or animals. Emission load is equally split throughout the year assuming that the wind pattern, meteorology are same throughout the year in the absence of better data.  Acreage data in Pune district was taken from Agricultural Marketing Committee of Pune.

The methodology used in calculating unpaved road dust emissions are based on "Agricultural Unpaved Road Dust Emission Reference California Air Resources Board Area Sources Emission Estimation Methodology Section 7.11, P. Gaffney, 1997 ". Particulate emissions from unpaved road dust are computed by multiplying a specific emission factor by an activity factor. For our purpose we have taken the California default emission factor as suggested by Mr. Patrick Gaffney, ARB, as 2.0 lbs PM10/VMT  (ref. California Air Resources Board San Joaquin Valley SIP, P. Gaffney, 1997). For the activity data we have taken "Acrage data available from Agricultural marketing committee, Pune, and 175 VMT per 40 acres available from Agricultural Unpaved Road Dust Emission, Reference

California Air Resources Board Area Sources Emission Estimation Methodology

Section 7.11, P. Gaffney, 1997".

The particulate dust emissions from agricultural unpaved road in each region using the following equation.

Emissionsunpaved_agri =Emission Factor unpaved_agri x VMT unpaved_agri Emission Factor (lb/VMT). =2.0 lbs PM10/VMT (ref. California Air Resources Board San Joaquin Valley SIP, P. Gaffney, 1997)

Assumptions:

- Emission factor is based on ARB data for agricultural unpaved roads.

- Emission factor assumed as 2 pound per VMT.

- Emission load is equally split throughout the year assuming that agricultural dust occur throughout the year in the absence of better data.

# Emission Factors

| PM10 |  |
| --- | --- |
| Emission Factor | 0.9072 |
|  | Kg/ VMT |

# Activity Data

Activity Data for agricultural Dust_Unpaved Road Estimates

| 2003 Annual VKT/ year in agricultural areas |  |
| --- | --- |
| Region | Grand Total VKT/year |
| 1 | 352,025.3 |
| 2 | 103,377.9 |
| 3 | 229,728.6 |
| 4 | 486,484.0 |
| 5 | 238,512.3 |
| 6 | 237,161.0 |
| 7 | 580,402.5 |
| 8 | 360,809.0 |
| 9 | 149,323.6 |
| 10 | 212,836.8 |
| 11 | 174,999.1 |
| 12 | 672,293.9 |
| 13 | 65,540.2 |

# Results

## Emission Estimates

| 2003 PM10 Emission Estimates (kg/year) |  | 2003 PM10 Emission Estimates (tons/year) |  |
| --- | --- | --- | --- |
| Region | Land cultivated by animals |  |  |
| 1 | 319,357.3 | 319.36 |  |
| 2 | 93,784.4 | 93.78 |  |
| 3 | 208,409.8 | 208.41 |  |
| 4 | 441,338.3 | 441.34 |  |
| 5 | 216,378.4 | 216.38 |  |
| 6 | 215,152.4 | 215.15 |  |
| 7 | 526,541.1 | 526.54 |  |
| 8 | 327,325.9 | 327.33 |  |
| 9 | 135,466.3 | 135.47 |  |
| 10 | 193,085.5 | 193.09 |  |
| 11 | 158,759.2 | 158.76 |  |
| 12 | 609,905.0 | 609.91 |  |
| 13 | 59,458.1 | 59.46 |  |
| Total annual kg per year | 3,504,961.8 | Grand Total tons per year | 3,504.96 |

Figure 1:2003 Agricultural Operations Emissions

# Unresolved Issues:

- Emission factor based on ARB data for agricultural unpaved road dust emissions.

- Emission factor assumed as 2 pounds per VMT.

- Emission load is equally split throughout the year assuming that agricultural operations from unpaved road dust emissions occur throughout the year in the absence of Specific VMT activities, such there might be more emissions from unpaved roads after harvest or during movement of goods, etc.

- Also the activity data is taken from ABR database as 175 VMT per 40 acres/year in the absence of specific data.

# Further Improvements

- Possibility of developing local emission factors for unpaved roads.

- Seasonal activity data.

- Activity data can be improved by replacing assumptions by actual data collection.