Agricultural Wind blown Dust

(Revised March 2004)

EMISSION INVENTORY SOURCE CATEGORY Miscellaneous Processes / Farming Operations

EMISSION INVENTORY CODES (CES CODES) AND DESCRIPTION _________________________________ (Agricultural Wind Blown Dust)

# METHODS AND SOURCES

The land preparation source category includes estimates of the airborne soil particulate emissions produced by wind flow across the field. The methodology used is based on “ARB Resources- Section 7.12, Wind blown Dust -Agricultural Areas EMISSION INVENTORY SOURCE CATEGORY (Miscellaneous Processes / Fugitive Windblown Dust)”. Dust emissions are produced by the mechanical disturbance of the soil by the wind. In the present inventory Emission load is equally split throughout the year assuming that PM emissions occur throughout the year equally in the absence of better data. The present methodology used in California uses a complicated equation requiring many inputs. Because of the lack of sufficient data, as per the suggestion of Mr. Patrick Gaffney, ARB, we have used the default San Joquain Valley, California, suspended particulate matter emission factors (in English tons per acre per year) averaged for all the counties. We had to convert Suspended PM emissions factor (English tons per acre per year) by a factor of 0.5 to arrive at PM10  Emissions factor.  In India it is a common practice to keep the agri vegetative cover for some time before burning or using it as a cattle feed, which is not the case with California. Hence we multiplied the emission factor value by a factor of 1/4. This is an assumption, which may or may not hold good actually. There is a need to substantiate this assumption by field data collection.  Acreage data in Pune district was taken from Agricultural Marketing Committee of Pune.

The particulate wind blown dust emissions from agricultural land preparation are estimated in each region (taluka) using the following equation.

Emissions Wind Blown_Agri=Emission  Factor WInd Blown_Agri x Acres agri

Assumptions:

- Emission factor is   0.001052 tons per acre per year, which is the default emission factor for SJV averaged over all the counties.

- we have assumed a longer vegetative coverage in India after the harvest, hence we have multiplied the above emission factor by a factor of 1/4.

- Emission load is equally split throughout the year assuming that agricultural operations occur throughout the year in the absence of better data.

# Emission Factors

PM10 emission factor:  0.001052 tons per acre per year.

# Activity Data

Activity Data for agricultural  wind blown Dust Estimates

|  | 2003 Annual Acres |
| --- | --- |
| Region | Total Area in Acres |
| 1 | 128,740.7 |
| 2 | 37,806.8 |
| 3 | 84,015.0 |
| 4 | 177,914.2 |
| 5 | 87,227.4 |
| 6 | 86,733.2 |
| 7 | 212,261.5 |
| 8 | 131,953.0 |
| 9 | 54,609.8 |
| 10 | 77,837.4 |
| 11 | 63,999.7 |
| 12 | 245,867.5 |
| 13 | 23,969.0 |

# Results

## Emission Estimates

| 2003 PM10 Emission Estimates (tons/year) |  |
| --- | --- |
| Region | Total Agri Wind Blown Dust Emissions |
| 1 | 135.5 |
| 2 | 39.8 |
| 3 | 88.4 |
| 4 | 187.2 |
| 5 | 91.8 |
| 6 | 91.3 |
| 7 | 223.4 |
| 8 | 138.9 |
| 9 | 57.5 |
| 10 | 81.9 |
| 11 | 67.4 |
| 12 | 258.7 |
| 13 | 25.2 |
| Total annual tons per year | 1,486.9 |

Figure 1:2003 Agricultural wind   blown dust Emissions

# Further Improvements

- Possibility of developing local emission factors.

- Possibility of developing  soil specific or taluka specific emission factors.

- Possibility of inclusion of meteorological data such as no. of rainy days, wind patter, wind speed, moisture content of soil, etc in the emission factor equation.

- Activity data can be improved by replacing assumptions by actual data collection.