AGRICULTURAL TRACTOR EMISSION

(Revised March 2004)

EMISSION INVENTORY SOURCE CATEGORY Miscellaneous Processes / Farming Operations

EMISSION INVENTORY CODES (CES CODES) AND DESCRIPTION _________________________________ (Agricultural Tractor Emission)

# METHODS AND SOURCES

The Agricultural Tractor Emission source category includes estimates of the tail pipe particulate emissions produced during the preparation of agricultural lands using tractors for planting and after harvest activities. Emission load is equally split throughout the year assuming that agricultural operations occur throughout the year in the absence of a crop calendar.  Acreage data in Pune district was taken from Agricultural Marketing Committee of Pune. The information on the number of passes was given by Mr. Harshvardhan Dhavan, Masters Student, UoP.

Particulate emissions from tractor operation are computed by specific emission factor as shown below by an appropriate activity factor. The equations for emission factor and the constants used therein are based on EPA NONROAD2002 document- EPA420-P-02-016 November 2002 R-009b.

# Emission Factor Equation:

EF adj PM = EF ss*TAF*DF-S PM adj                 ……………………………….Equation (1)

EFadj PM = Adjusted particulate emission factor (g/hp-hr)

EF ss = Steady State zero hour emission factor (g/hp-hr)

TAF= Transient Adjustment Factor

DF= Deterioration factor (unitless) 

DF=1+A* (Age Factor)^b

A= Fraction of median life expended=0.473   (Table A4. Deterioration Factors for Nonroad Diesel Engines, EPA420-P-02-016 November 2002 R-009b) ;

Age Factor= (Cumulative hours *load factor)/median life at full load

A,b constant for given technology type ; for CI engines b=1

Load Factor is assumed as 0.6;

Median Life at full load is assumed as 5000 hrs.

SPMadj=Sulphur Adjustment factor; has been neglected for this purpose assuming that it is too small to influence the emission factor value.

The production data from the tractor manufacturers (in four categories of power) was totaled and were assigned to EPA power category (>25-50hp and >50-100hp) and percentage in each category was calculated. From this percentage in each category, the number of tractors in each of these two categories of power was calculated from the registration data.

Since individual horse power ratings of each tractors was not known, a weighted average of power was calculated from the production volume and the power rating of that category.

# Activity data:

Although the tractor operation duration is dependent on area and number of passes per acre, which is again dependent on type of crop, for our purpose due to the lack of data we have assumed the number of passes as 8 (which is correct in case of wheat) for all other crops except sugar cane where the data provided by Mr. Dhavan was used.

In order to calculate the hours of operation of tractor by category the following procedure was done.

- From the area of cultivated land in sq. m, and assuming a fraction of tractor cultivation as 5% and plough width of 2 m, the length of travel of tractor is calculated.

- Speed of tractor operation was assumed as 10 kph. Dividing the result obtained in step 1 by speed of tractor, we obtain no. o hours of operation per year.

- The result obtained above was multiplied by the number of passes in a year to obtain no. of hours of operation.

- tractor population was taken from the registration data

Assumptions:

- Emission factor is based on EPA data for agricultural tractors.

- EF based on ISO C1 8 mode cycles. Operating cycle in India may be different

- Sensitivity of EF to fuel sulphur levels. Default Sulphur assumed as 0.2%, whereas in India, 0.05 % became available on 1st April 2003

- Engine technology may be different for Indian Tractors

- TAF is assumed as 1.23

- Age factor 0.473

# Emission Factors

1. Actual equation:					EF adj PM = EF ss*TAF*DF-S PM adj

2. Equation used for calculations			EF adj PM = EF ss*TAF*DF

3. ‘A’ value						0.473	 (EPA table)

4. Total no. of hours of operation per tractor	1568400 (From calculations)

5. Total no. of registred tractors in Pune District	18428	(Reg. Data)

6. Avg. no. of hours of operation per tractor	85.11	 (4/5)

7. Load Factor					0.6	(assumed)

8. Median Life at full Load				5000	(assumed)

9. Age Factor						0.0102	 (step6*step7/step8)

10. DF							1.0048	 (1+step9*step3)

11. EFss						>25-50		>50-100

0.3389		0.473

12. Transient Adjustment Factor(TAF)		1.23	(EPA table)

13. Tractor Population as per EPA category	>25-50hp= 13316	(weighted avg.)								>50-100hp=5112

14. EFPMadj						=EFss*TAF*DF

| PM10 Emission Factor | tractor category 1 | tractor category 2 |
| --- | --- | --- |
|  | 0.41886 | 0.5856 |
|  | g/bhp-hr | g/bhp-hr |

# Activity Data

|  | 2004 Annual activity data |  |
| --- | --- | --- |
| Region | composite activity data for tractor category 1 in hours-hp | composite activity data for tractor category 2 in hours-hp |
| 1 | 3,991,067 | 2,187,568 |
| 2 | 1,172,041 | 642,414 |
| 3 | 2,604,535 | 1,427,587 |
| 4 | 5,515,486 | 3,023,126 |
| 5 | 2,704,120 | 1,482,172 |
| 6 | 2,688,800 | 1,473,774 |
| 7 | 6,580,282 | 3,606,758 |
| 8 | 4,090,652 | 2,242,152 |
| 9 | 1692947.908 | 927931.8362 |
| 10 | 2413025.299 | 1322617.776 |
| 11 | 1984043.023 | 1087485.727 |
| 12 | 7622095.784 | 4177792.656 |
| 13 | 743058.584 | 407282.2991 |

# Results

## Emission Estimates

| 2003 PM10 Emission Estimates (kg/year) |  |  |  |
| --- | --- | --- | --- |
| Region | Tractor category 1 | Tractor category 2 | Total |
| 1 | 1,671.7 | 1,278.9 | 2,950.6 |
| 2 | 490.9 | 375.6 | 866.5 |
| 3 | 1,090.9 | 834.6 | 1,925.5 |
| 4 | 2,310.2 | 1,767.3 | 4,077.5 |
| 5 | 1,132.6 | 866.5 | 1,999.1 |
| 6 | 1,126.2 | 861.6 | 1,987.8 |
| 7 | 2,756.2 | 2,108.5 | 4,864.7 |
| 8 | 1,713.4 | 1,310.8 | 3,024.2 |
| 9 | 709.1 | 542.5 | 1,251.6 |
| 10 | 1,010.7 | 773.2 | 1,783.9 |
| 11 | 831.0 | 635.7 | 1,466.8 |
| 12 | 3,192.6 | 2,442.3 | 5,634.9 |
| 13 | 311.2 | 238.1 | 549.3 |
|  |  | Grand Total foor Pune District | 32,382.5 |

Figure 1:2003 Agricultural Operations Emissions

# Further Improvements

- Possibility of developing local emission factors.

- Power based EF to Model based EF.

- Activity data can be improved by replacing assumptions by actual data collection.