| Emission Inventory Source Methodology Summary Form |  |
| --- | --- |
| Source Name: | Rural and Urban Agricultural Pumps |
| Source ID Code: |  |
| Source Description: | Tail pipe Particulate matter emissions caused by Industrial and commercial gensets using diesel fuel for power generation and/or other operations. This method does not include sulphur correction factors for EF. Also it does not include the thermal generators. |
| Staff: | Leena Pandey, leena@siamindia.com, SIAM Prashant Pawar , pawar.ecl@araiindia.com ARAI, |
| Update History: | March 2004 |
| Methodology: | U.S. EPA 420-P-02-016 November 2002 |
| Inputs – EF: | EF adj PM = EF ss*TAF*DF-S PM adj EFadj PM = Adjusted particulate emission factor (g/hp-hr) EF ss = Steady State zero hour emission factor (g/hp-hr) TAF= Transient Adjustment Factor DF= Deterioration factor (unitless) DF=1+A* (Age Factor)^b A= Fraction of median life expended=0.473 ; Age Factor= (Cumulative hours *load factor)/median life at full load A,b constant for given technology type ; for CI engines b=1 Load Factor is assumed as 0.6; Median Life at full load is assumed as 5000 hrs. SPMadj=Sulphur Adjustment factor; has been neglected for this purpose |
| Inputs – Activity: | Hrs. of Usage per day (Assumed), No. of Agricultural Pumps (Source :MSEB) Aggregate power = weighted average of 3-5 HP & 5-10 HP |
| Estimated Emissions: | Total emissions for entire Pune District are estimated to be 12.52 tons/year |
| Date Loaded to DB: | 23-Mar-2004 |
| Unresolved Issues: | EF based on ISO C1 8 mode cycles. Operating cycle in India may be different Sensitivity of EF to fuel sulphur levels. Engine technology may be different for Indian agricultural Pumps. Emission factor is assumed as the average for different power categories. |
| Planned Improvements: | Possibility of developing local emission factors. Power based EF to Model based EF. Activity data can be improved by replacing assumptions by actual data collection. |