| Emission Estimation Diesel Locomotives - PM10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Contacts: | Mohit Davli, CDAC, Pune; Mangesh.S. Dighe, PMC, Pune |  |  |  |
| Back ground data |  |  |  |  |
| Track length Pune area | 10 | km | Pune Rly station |  |
| Trains-Express (trips) | 110 | per day | Part Diesel/elec |  |
| Trains-Passenger (trips) | 26 | per day | Part Diesel/elec |  |
| Trains-Goods (trips) | 20 | per day | All diesel |  |
| Idling time | 20 | min | Change from elec to diesel |  |
| National Diesel consumption (2000-01) | 2,004,000,000 | ltrs/yr | Indian Railways website |  |
| National track length | 63560 | km | Indian Railways website |  |
| Emission Factor (PM)/PM10 | 0.0014 | kg/ltr | EPA-Mexico Inventory Vol. V |  |
| Diesel loco size (new WDM4 class) | 3400 | hp | Indian Railways website |  |
| Loco Fuel consumption (average) | 0.156 | kg/hp/hr | Indian Railways website |  |
| (includes moving + idling) |  |  |  |  |
| Emission from movement |  |  |  |  |
| Fuel consumption in Pune area | 315292.6369 | ltrs/yr | =National fuel * (pune track/national track) |  |
| PM emission from movement | 441.4096916 | kg/yr |  |  |
| Emission from Idling |  |  |  |  |
| Number of trains (diesel) | 81 | per day | See estimate below |  |
| Idling per day (all diesel trains) | 27 | hrs/day |  |  |
| Total idling time per year | 9855 | hrs/yr |  |  |
| Fuel consumption | 5227092 | kg/yr |  |  |
| Fuel consumption (volume based) | 5807880 | ltrs/yr | avg diesel specific gravity 0.9 kg/l |  |
| PM emission from idling | 8131.032 | kg/yr |  |  |
| Total Emission Diesel Loco | 8572.441692 | kg/yr |  |  |
| PEIC Code 8202000000 | 8.572441692 | Ton/yr |  |  |
| Number of trains per day |  |  |  |  |
| Express=110 total - 12 (known electric) = 98*0.5 (assume 50-50 diesel/electric) = 49 |  |  |  |  |
| Passenger = 26 - 2 (known electric) = 24*0.5 (50-50 diesel/electric) = 12 |  |  |  |  |
| Goods = 20 all diesel = 20 |  |  |  |  |
| Total |  |  |  | 81 |