| Emission Inventory Source Methodology Summary Form |  |
| --- | --- |
| Source Name: | Industrial and Non Industrial Generators |
| Source ID Code: |  |
| Source Description: | Tail pipe Particulate matter emissions caused by Industrial and Non Industrial Generators using diesel fuel for power generation and/or other operations. This method does not include sulphur correction factors for EF. |
| Staff: | Leena, SIAM ; Prashant, ARAI |
| Update History: | March 2004 |
| Methodology: | U.S. EPA 420-P-02-016 November 2002 |
| Inputs – EF: | EF adj PM = EF ss*TAF*DF-S PM adj EFadj PM = Adjusted particulate emission factor (g/hp-hr) EF ss = Steady State zero hour emission factor (g/hp-hr) TAF= Transient Adjustment Factor DF= Deterioration factor (unitless) DF=1+A* (Age Factor)^b A= Fraction of median life expended=0.473 ; Age Factor= (Cumulative hours *load factor)/median life at full load A,b constant for given technology type ; for CI engines b=1 Load Factor is assumed as 0.6; Median Life at full load is assumed as 5000 hrs. SPMadj=Sulphur Adjustment factor; has been neglected for this purpose |
| Inputs – Activity: | the number of generators for Industrial, no. of hours of operation, average KVA based on capacity utilisation |
| Estimated Emissions: | Total emissions for entire Pune District are estimated to be 57.21 tons/year. |
| Date Loaded to DB: | 23-Mar-2004 |
| Unresolved Issues: | EF based on ISO C1 8 mode cycles. Operating cycle in India may be different Sensitivity of EF to fuel sulphur levels. Default Sulphur assumed as 0.2%, whereas in India, 0.05 % became available on 1st April 2003 Engine technology may be different for Indian generators. Emission factor is assumed as taken as the average for different power categories. |
| Planned Improvements: | Possibility of developing local emission factors. Power based EF to Model based EF. Activity data can be improved by replacing assumptions by actual data collection. |