| # Emission Inventory Source Methodology Summary Form |  |
| --- | --- |
| Source Name: | Unpaved Road Dust (Non agricultural) |
| Source ID Code: | 6450000000 |
| Source Description: | Particulate matter emissions caused by vehicle travel on unpaved roads, which entrains the dust on the roads into the air. |
| Staff: | Ajay (NEERI) – ajayrojha@rediffmail.com Prashant (ARAI) – pawar.ecl@araiindia.com Deepak Kapoor (IIT, Bombay)– deepak.kapoor@iitb.ac.in |
| Update History: | March 2004 |
| Methodology: | U.S. EPA AP-42, Section 13.2.1 March 1993 |
| Inputs – EF: | E = k (sL/2)0.65 (W/3 )1.5 (1-P/4N) E = particulate emission factor (having units matching the units of k) k = particle size multiplier for particle size range and units of interest Source: AP-42 methodology sL = road surface silt loading (grams per square meter) (g/m2) sL have been selected based on the worst case scenarios for paved road dust loadings as stated in AP – 42. Source: EPA defaults, roads subdivided into high and low traffic roads W = average weight (tons) of the vehicles traveling on the road Source: Engineering estimate for Pune vehicle fleet P = number of “wet” days with at least 0.254 mm (0.01 in) of precipitation during the averaging period Source: Pune average 10 year meteorological data from airport N = number of days in the averaging period (e.g., 365 for annual, 91 for seasonal, 30 for monthly) |
| Inputs – Activity: | It is assumed that 20% of the total road is unpaved including the shoulders as gathered from the actual data collected from PMC. However, the unpaved road conditions in Pune is much comparable to the moderate dust laden conditions as also the total vehicle movement on the unpaved road may only be a fraction of the total vehicle considered. Data source: Pune IVEM model study further subdivided into ward-wise data based on area. |
| Estimated Emissions: | Total emissions for entire Pune region are estimated to be 5833 Tons/year. Emissions subdivided by region are shown in the emissions table. |
| Date Loaded to DB: | 23-Mar-2004 |
| Unresolved Issues: | The average weight of the vehicles has been calculated by weighted average means. The average distance traveled by each vehicle category may not be representative since the total vehicles have been considered instead of the actual count on the road Average weight for final calculations may not be representative of individual vehicle type Actual silt and moisture content in Pune region has to be estimated |
| Planned Improvements: | Perform unpaved road silt loading measurements in Pune Improve vehicle traffic estimates Evaluate road network to evaluate if additional road categories are warranted |